Check: bending moment steel beam (axis My)

Profile HEB300 with steel quality S355 is loaded with a bending moment of 328.43 kNm (axis My). Additionally a shear force of 829.26 kN (axis Vz). The resistance is calculated as follows, using cross-section class 1 or 2:

First, the shear area is determined:

Av=max(A−2·b·tf+(tw+2·r)·tf;η·hw·tw)=

max(14910.68−2·300.00·19.00+(11.00+2·27.00)·19.00;1.00·262.00·11.00)=

4745.68 mm2 (EN 1993-1-1:2005 6.18suba)

The shear resistance is calculated as:

Vpl,Rd=Av·(fy/3)γM0=4745.68·(355.00/3)1.00=972672.43 N (EN 1993-1-1:2005 6.18)

The reduction factor for bending moment resistance is defined as:

ρ=0.50 − (EN 1993-1-1:2005 6.29rho)

This gives a reduced yield strength of:

fy,red=(1−ρ)·fy=(1−0.50)·355.00=178.50 MPa (EN 1993-1-1:2005 6.29)

The bending moment resistance with reduced yield strength is:

Mc,Rd=Wpl·fyγM0=1869026.14·178.501.00=333621355.30 Nmm (EN 1993-1-1:2005 6.13)

The unity check is calculated as follows:

CHECK→MEdMc,Rd≤1→328430025.00333621355.30≤1→OK (EN 1993-1-1:2005 6.12)

The check for bending moment satisfies the requirements.